EXTRON  DXP HDMI 系列 HDMI矩阵切换器

Extron DXP HDMI 系列是高性能的数字矩阵切换器，用于将 HDMI 信号从多个信号源路由至任意或全部 8 个具有 HDMI 功能的显示设备。这些与 HDMI 1.3 兼容的矩阵切换器支持高达 1920x1200 和 HDTV 1080p/60 的分辨率，并且符合 HDCP 标准，允许将一个信号源同时分配至一个或多个兼容的显示设备。DXP HDMI 系列提供 4x4、4x8、8x4 和 8x8 的不同尺寸，设计用于轻松地集成到需要可靠 HDMI 信号路由的应用环境中。它们还具有一些 Extron 矩阵切换器常见的便利特性，例如 QS-FPC? – 快速切换前面板控制器、全局预设、IP Link? 以太网控制等。

主要特性

|  | 可提供 4x4、4x8、8x4 和 8x8 的输入/输出尺寸 |
| --- | --- |
|  | 兼容 HDMI 1.3 |
|  | 符合 HDCP 标准 |
|  | EDID 管理器 (EDID Minder?) |
|  | 分辨率为 1920x1200 时，可对每路输入进行最远距离为 100' (30 m) 的自动电缆均衡 |
|  | 自动输出时钟恢复 |

| 型号 | 产品说明 | 产品编号 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DXP 44 HDMI | 4x4 HDMI 矩阵切换器板 | 60-880-01 |  |  |  |  |  |
|  | DXP 48 HDMI | 4x8 HDMI 矩阵切换器板 | 60-1010-01 |  |  |  |  |  |
|  | DXP 84 HDMI | 8x4 HDMI 矩阵切换器板 | 60-881-01 |  |  |  |  |  |
|  | DXP 88 HDMI | 8x8 HDMI 矩阵切换器板 | 60-882-01 |  |  |  |  |  |

Extron DXP HDMI 系列是高性能的数字矩阵切换器，用于将 HDMI 信号从多个信号源路由至任意或全部 8 个具有 HDMI 功能的显示设备。这些与 HDMI 1.3 兼容的矩阵切换器支持高达 1920x1200 和 HDTV 1080p/60 的分辨率，并且符合 HDCP 标准，允许将一个信号源同时分配至一个或多个兼容的显示设备。DXP HDMI 系列提供 4x4、4x8、8x4 和 8x8 的不同尺寸，设计用于轻松地集成到需要可靠 HDMI 信号路由的应用环境中。它们还具有一些 Extron 矩阵切换器常见的便利特性，例如 QS-FPC? – 快速切换前面板控制器、全局预设、IP Link? 以太网控制等。

| Video |  |
| --- | --- |
| Routing |  |
| DXP 44 HDMI | 4 x 4 matrix |
| DXP 48 HDMI | 4 x 8 matrix |
| DXP 84 HDMI | 8 x 4 matrix |
| DXP 88 HDMI | 8 x 8 matrix |
| Signal type | TMDS digital RGB and single-link DVI digital video signals are supported |
| Digital video | RGB digital video (DVI and HDMI standards) or Y, Cr, Cb digital component video (HDMI), actively buffered (supports all single-link DVI and HDMI [if using an optional adapter] standards from 640x480 @ 60 Hz to 1600x1200 @ 60 Hz computer video and HDTV 480p, 720p, 1080i, 1080p) |
| NOTE; The DXP HDMI Series switchers are compatible with HDMI 1.3.. |
| Digital audio | Supports HDMI audio transmitted through the RGB and Y, Cr, Cb lines, actively buffered. |
| EDID and DDC | Supports Extended Display Identification Data (EDID) and Display Data Channel (DDC) data using DVI and HDMI standards. EDID and DDC signals are actively buffered. |
| HDCP | Compliant with High-bandwidth Digital Content Protection (HDCP) using DVI and HDMI 1.3 standards |
| HPD | Supports hot plug detection (HPD) of display as a pass-through signal. |
| Gain | Unity |
| Resolution range | Up to 1080p (HDTV) or 1920x1200 (the highest resolution of the single-link DVI standard) @ 60 Hz |
| Maximum data rate | 6.75 Gbps (2.25 Gbps per color) |
| Maximum pixel clock | 225 MHz |
| Standards | DVI 1.0, HDMI 1.3 |
| Switching speed | 200 ns, max. |
| Video input |  |
| Number/signal type | 4 or 8 (depending on model) digital RGB (TMDS) HDMI (or single-link DVI-D*) |
| Connectors | 4 or 8 female HDMI type A (digital only) |
| Nominal level |  |
| Digital video | 1.2 Vp-p |
| DDC (Display Data Channel) | 5.0 Vp-p (TTL) |
| Minimum/maximum level | 0.5 V to 1.0 Vp-p with no offset |
| Impedance | 100 ohms |
| Return loss | <-15 dB @ 1 MHz to 1.5 GHz |
| TDR rise time (10%-90%) | 75 ps |
| Equalization | Automatic |
| Input cable length | >100' (30 m) at 1920x1200 @ 48, 50, or 60 Hz; or 1080p; 8 bit color |
| NOTE; The transmission distance varies depending on the signal resolution and on the type of cable, graphic card, and display used in the system.. |
| Video output |  |
| Number/signal type | 4 or 8 (depending on model) digital RGB |
| Connectors | 4 or 8 female HDMI type A |
| Nominal level | 1.2 Vp-p |
| Minimum/maximum level(s) | 0.5 V to 1.0 Vp-p with no offset (follows input) |
| Impedance | 100 ohms |
| Return loss | <-15 dB @ 1 MHz to 1.5 GHz |
| DC offset | ±500 mV maximum with input at 0 offset |
| Rise and fall time (20-80%) | 0.6 ns |
| Re-clocking | Automatic |
| Peripheral device power | 250 mA per output |
| Control/remote — switcher |  |
| Serial control port | (1) RS-232, 9-pin female D connector (1) RS-232, front panel 2.5 mm mini stereo jack |
| Baud rate and protocol | 9600 to 115200 baud, 9600 baud (default), 8 data bits, 1 stop bit, no parity |
| Serial control pin configurations |  |
| 9-pin D connector | 2 = Tx, 3 = Rx, 5 = GND |
| Mini stereo jack | Tip = Tx, ring = Rx, sleeve = GND |
| Ethernet control port | (1) RJ-45 female connector |
| Ethernet data rate | 10/100Base-T, half/full duplex with autodetect |
| Ethernet protocol | ARP, ICMP (ping), IP, TCP, DHCP, HTTP, Telnet |
| Default settings | Link speed and duplex level = autodetected IP address = 192.168.254.254 Subnet mask = 255.255.0.0 Gateway = 0.0.0.0 DHCP = off |
| Program control | Extron control/configuration program for Windows? Extron Simple Instruction Set (SIS?) Microsoft? Internet Explorer? ver. 6 or higher, Telnet |
| General |  |
| Power | 100 VAC to 240 VAC, 50-60 Hz, internal; fully loaded 8 x 8 unit |
| 115 VAC, 60 Hz | 48 watts |
| 240 VAC, 50 Hz | 48 watts |
| Temperature/humidity | Storage: -40 to +158 °F (-40 to +70 °C) / 10% to 90%, noncondensing Operating: +32 to +122 °F (0 to +50 °C) / 10% to 90%, noncondensing |
| Cooling | Fan, air flows right to left (as viewed from front) |
| Thermal dissipation, full load |  |
| 115 VAC, 60 Hz | 129.7 BTU/hr |
| 240 VAC, 50 Hz | 126.3 BTU/hr |
| Mounting |  |
| Rack mount | Yes, 2U high |
| Enclosure type | Metal |
| Enclosure dimensions | 3.5" H x 17.5" W x 12.0" D (2U high, full rack wide) (8.9 cm H x 44.4 cm W x 30.5 cm D) (Depth excludes connectors. Width excludes integrated rack ears.) |
| Product weight | 10.0 lbs (4.5 kg) |
| Shipping weight | 15 lbs (7 kg) |
| Vibration | ISTA 1A in carton (International Safe Transit Association) |
| Regulatory compliance |  |
| Safety | CE, c-UL, UL |
| Compliances | CE, C-tick, FCC Class A, ICES, VCCI |
| MTBF | 30,000 hours |
| Warranty | 3 years parts and labor |