EXTRON  FOX SW8 切换器

Extron FOX SW8 是一款 8 路输入光纤切换器，可以通过单根光纤对输入的光纤视音频和 RS-232 控制信号进行切换和长距离传输。它提供了 8 路光纤输入和 1 路光纤输出，均用于通过单根光纤链路长距离连接至 FOX 系列光纤发送器、接收器、分配放大器或矩阵切换器。FOX SW8 支持高达 4.25 Gbps 的速率，接受并切换标清视频、高分辨率计算机视频、DVI-D 和多速率 SDI 信号。FOX SW8 专为视音频系统而设计，包括了很多便于集成的特性，如 RS-232 控制、机架安装功能、输出静音功能以及 8 路输入中的 7 路可进行输入环通。

主要特性

|  | 与所有的 FOX 系列发送器和接收器相兼容 |
| --- | --- |
|  | 通过单根光纤极长距离地切换和传输光纤视音频和 RS-232 控制信号 |
|  | 8 路带环通的光纤输入 |
|  | 输出屏蔽控制 |
|  | 光纤链路丢失时的警报通知 |

|  | 型号 | 产品说明 | 产品编号 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FOX SW8 MM | 多模 | 60-984-01 |  |  |  |  |  |
|  | FOX SW8 SM | 单模 | 60-984-02 |  |  |  |  |  |

Extron FOX SW8 是一款 8 路输入光纤切换器，可以通过单根光纤对输入的光纤视音频和 RS-232 控制信号进行切换和长距离传输。它提供了 8 路光纤输入和 1 路光纤输出，均用于通过单根光纤链路长距离连接至 FOX 系列光纤发送器、接收器、分配放大器或矩阵切换器。FOX SW8 支持高达 4.25 Gbps 的速率，接受并切换标清视频、高分辨率计算机视频、DVI-D 和多速率 SDI 信号。FOX SW8 专为视音频系统而设计，包括了很多便于集成的特性，如 RS-232 控制、机架安装功能、输出静音功能以及 8 路输入中的 7 路可进行输入环通。

| Optical specifications |  |
| --- | --- |
| NOTE; These switchers are class 1 laser products. They meet the safety regulations of IEC-60825, FDA 21 CFR 1040.10, and FDA 21 CFR 1040.11.. |
| Number/type |  |
| Input | 8 singlemode or multimode fiber optic |
| Output | 1 singlemode or multimode fiber optic with 7 loop throughs for inputs 1-7 |
| Connectors | 8 LC connectors |
| Operating distance |  |
| Singlemode | 30 km (18.75 miles) with singlemode (SM) cables with an Extron singlemode Tx/Rx unit |
| Multimode | 300 m (985') with 62.5 μm multimode (MM) cables with an Extron multimode Tx/Rx unit 1 km (3280') with 50 μm multimode (MM) cables with an Extron multimode Tx/Rx unit 2 km (6561') with 50 μm 2000 MHz bandwidth laser multimode cable with an Extron multimode Tx/Rx unit |
| NOTE; Operating distance is approximate. These are typical distances. The maximum distance may be greater than these typical numbers depending on factors such as fiber type, fiber bandwidth, connector splicing, losses, modal or chromatic dispersion, environmental factors, and kinks.. |
| Nominal peak wavelength | 850 nm for MM model, 1310 nm for SM model |
| Data rate | 4.25 Gbps |
| Transmission power |  |
| Singlemode | -5 dBm, typical |
| Multimode | -5 dBm, typical |
| Maximum receiver sensitivity |  |
| Singlemode | -18 dBm, typical |
| Multimode | -12 dBm, typical |
| Optical loss budget |  |
| Singlemode | 13 dB, maximum |
| Multimode | 7 dB, maximum |
| Control/remote |  |
| Serial control ports | 1 RS-232, 3.5 mm captive screw connector, 3 pole (rear panel) 1 RS-232, 2.5 mm mini stereo jack (front panel) |
| Baud rate and protocol | 9600 baud, 8 data bits, 1 stop bit, no parity |
| Serial control pin configurations | Captive screw connector: 1 = Tx, 2 = Rx, 3 = GND Mini stereo jack: tip = Tx, ring = Rx, sleeve = GND |
| Program control | Extron control/configuration program for Windows? Extron Simple Instruction Set (SIS?) |
| General |  |
| Power | 100 VAC to 240 VAC, 50-60 Hz, 20 watts, internal |
| Temperature/humidity | Storage: -40 to +158 °F (-40 to +70 °C) / 10% to 90%, noncondensing Operating: +32 to +122 °F (0 to +50 °C) / 10% to 90%, noncondensing |
| Cooling | Convection, vents on top and sides |
| Mounting |  |
| Rack mount | Yes, with optional 1U rack shelf |
| Furniture mount | Yes, with optional under-desk mounting kit or through-desk mounting kit |
| Enclosure type | Metal |
| Enclosure dimensions | 1.7" H x 8.7" W x 9.5" D (1U high, half rack wide) (4.3 cm H x 22.1 cm W x 24.1 cm D) (Depth excludes connectors.) |
| Product weight | 2.4 lbs (1.1 kg) |
| Shipping weight | 4 lbs (2 kg) |
| Vibration | ISTA 1A in carton (International Safe Transit Association) |
| Regulatory compliance |  |
| Safety | CE, c-UL, UL |
| EMI/EMC | CE, C-tick, FCC Class A, FDA Class 1, ICES, VCCI |
| MTBF | 30,000 hours |
| Warranty | 3 years parts and labor |