EXTRON  RGB-DVI 300转换器

Extron RGB-DVI 300 是一款高性能 RGB 至 DVI 图像解析度转换器，可以将收到的模拟分量视频和 RGBHV 信号转换为 DVI-D 信号。它接受 HDTV 信号和分辨率高达 1920x1200 的计算机视频信号，并且提供多种输出分辨率，最高可达 1920x1200，包括 HDTV 1080p/60。该产品容纳于一个紧凑、小巧的外壳中，可以放置在平板显示器后面，并且提供了几种方便集成和增强系统操作的特性，包括自动输入存储、自动图像（Auto-Image?）设置和 RS-232 串行控制。

RGB-DVI 300

RGB 至 DVI 图像解析度转换器

主要特性

|  | 将 RGB 和 HDTV 分量视频解析度转换为 DVI |
| --- | --- |
|  | 高达 1920x1200 的 80 种可选择的输出分辨率，包括 HDTV 1080p/60 |
|  | 自动图像（Auto-Image?）设置 |
|  | 自动输入存储 |
|  | 图像控制特性，包括亮度、对比度、细节调节以及横向和纵向定位、大小和缩放调整 |

| 型号 | 产品说明 | 产品编号 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RGB-DVI 300 | RGB 至 DVI 图像解析度转换器 | 60-906-01 |  |  |  |

Extron RGB-DVI 300 是一款高性能 RGB 至 DVI 图像解析度转换器，可以将收到的模拟分量视频和 RGBHV 信号转换为 DVI-D 信号。它接受 HDTV 信号和分辨率高达 1920x1200 的计算机视频信号，并且提供多种输出分辨率，最高可达 1920x1200，包括 HDTV 1080p/60。该产品容纳于一个紧凑、小巧的外壳中，可以放置在平板显示器后面，并且提供了几种方便集成和增强系统操作的特性，包括自动输入存储、自动图像（Auto-Image?）设置和 RS-232 串行控制。

| RGB-DVI 300, RGB-HDMI 300 A |  |
| --- | --- |
| Video input |  |
| Number/signal type | 1 RGBHV, RGBS, RGsB, HDTV component video (Y, R-Y, B-Y) |
| Connectors | 1 female 15-pin HD |
| Nominal level | 1.0 Vp-p for Y of component video 0.7 Vp-p for RGB and for R-Y and B-Y of component video |
| Minimum/maximum levels | Analog: 0.0 V to 2.0 Vp-p with no offset |
| Impedance | 75 ohms |
| Horizontal frequency | 30 kHz to 100 kHz |
| Vertical frequency | 24 Hz to 120 Hz |
| Resolution range | 640x480 to 1920x1200* or 480p, 576p, 720p, 1080i, or 1080p |
| NOTE; *Only the reduced blanking version of the 1920x1200 resolution is sampled at full bit rate.. |
| Formats | RGB and HDTV Y, R-Y, B-Y |
| Return loss | <-30 dB @ 5 MHz |
| Video processing |  |
| Digital sampling | 24 bit, 8 bits per color; 162 MHz standard |
| Colors | 16.78 million |
| Video output |  |
| Number/signal type |  |
| RGB-DVI 300 | 1 single link DVI-D |
| RGB-HDMI 300 A | 1 single link HDMI |
| Connectors |  |
| RGB-DVI 300 | 1 female DVI-I (digital output only) |
| RGB-HDMI 300 A | 1 female HDMI |
| Scaled resolutions | 640x4806,8,9, 800x6006,8,9, 852x4806,8,9, 1024x7686,8,9, 1024x8526,8,9, 1024x10246,8,9, 1280x7686,8,9, 1280x8006,8,9, 1280x10246,8,9, 1360x7656,8,9, 1360x7686,8,9, 1365x7686,8,9, 1365x10246,8,9, 1366x7686,8,9, 1400x9006,8,9, 1400x10506,8, 1600x12006,8, 1680x10506,8, 1920x12006,8, 2048x10801,2,3,4,5,6,7,8 480p7,8, 576p6, 720p3,4,5,6,7,8, 1080p1,2,3,4,5,6,7,8, 1080i6,7,8 |
|  | 1 = at 23.98 Hz, 2 = at 24 Hz, 3 = at 25 Hz, 4 = at 29.97 Hz, 5 = at 30 Hz, 6 = at 50 Hz, 7 = at 59.94 Hz, 8 = 60 Hz, 9 = 75 Hz; or automatic based on display device or current input rate |
| Formats | RGB and YCbCr digital video |
| Sync |  |
| Input type | RGBHV, RGBS, RGsB; bi-level YUVp component video, or tri-level HDTV component video |
| Output type | DVI-D or HDMI digital video |
| Standards | DVI 1.0, HDMI 1.2 |
| Input level | 2.75 V to 5.0 Vp-p for RGBHV or RGBS 0.6 Vp-p for component video tri-level sync 0.3 Vp-p for component video bi-level sync or RGsB |
| Input impedance | 10k ohms |
| Audio — RGB-HDMI 300 A |  |
| Frequency response | 20 Hz to 20 kHz, ±1.1 dB |
| THD + Noise | 0.03% @ 1 kHz at nominal level, 0 dB gain |
| S/N | >70 dB at rated maximum output |
| Audio input — RGB-HDMI 300 A |  |
| Number/signal type | 1 stereo, unbalanced |
| Connectors | (1) 3.5 mm mini stereo (tip-ring-sleeve) jack |
| Impedance | >10k ohms unbalanced, DC coupled |
| Nominal level | +4 dBu (1.23 Vrms), -10 dBV (316 mVrms) |
| Maximum level | +10.4 dBu, (unbalanced) at 1% THD+N |
| Input gain adjustment | -18 dBu to +10 dBu |
| NOTE; 0 dBu = 0.775 Vrms, 0 dBV = 1 Vrms, 0 dBV ≈ 2 dBu. |
| Audio output — RGB-HDMI 300 A |  |
| Number/signal type | 1 stereo, digital LPCM 48 kHz (embedded in HDMI output) |
| Connectors | 1 HDMI |
| Control/remote — scaler |  |
| Serial control ports | 1 RS-232, 3.5 mm captive screw connector, 3 pole (rear panel) 1 RS-232, 2.5 mm mini stereo jack (front panel) |
| Baud rate and protocol | 9600 baud, 8 data bits, 1 stop bit, no parity |
| Serial control pin configurations | 1 = Tx, 2 = Rx, 3 = GND |
| Program control | Extron control/configuration program for Windows? Extron Simple Instruction Set? (SIS?) |
| General |  |
| External power supply | 100 VAC to 240 VAC, 50-60 Hz, external; to 12 VDC, 2 A |
| Power input requirements | 12 VDC, 1 A |
| Temperature/humidity | Storage -40o to +158oF (-40o to +70oC) / 10% to 90%, noncondensing Operating +32o to +122oF (0o to +50oC) / 10% to 90%, noncondensing |
| Cooling | Convection, vents on top and sides |
| Thermal dissipation |  |
| RGB-DVI 300 | 24.5 BTU/hour |
| RGB-HDMI 300 A | 32.6 BTU/hour |
| Mounting |  |
| Rack mount | Yes, with optional rack shelf kit |
| Wall mount | Yes, with optional under-desk or through-desk mounting kit |
| Enclosure type | Metal |
| Enclosure dimensions | 1.0 H x 8.75" W x 6.0" D (half rack wide) (2.5 cm H x 22.2 cm W x 15.2 cm D) (Depth excludes connectors and knobs.) |
| Product weight | 1.6 lbs (0.7 kg) |
| Shipping weight | 4 lbs (2 kg) |
| Vibration | ISTA/NSTA 1A in carton (International Safe Transit Association) |
| Regulatory compliance |  |
| Safety | CE, c-UL, UL |
| EMI/EMC | CE, C-tick, FCC Class A, ICES, VCCI |
| Environmental | Complies with the appropriate requirements of WEEE. |
| MTBF | 30,000 hours |
| Warranty | 3 years parts and labor |