EXTRON  SW USB 切换器

Extron SW2 USB、SW4 USB 和 SW4 USB Plus 是 2 路和 4 路输入的 USB 切换器，允许在多个主机设备与多个 USB 外围设备之间进行切换。它们为在当今的视音频环境中实现兼容 USB 2.0 的切换提供了易于集成的解决方案。所具备的几种特性可确保轻松集成，包括端口状态指示、RS-232 直通、前面板安全锁定和多点控制。SW4 USB Plus 提供了主机和外围设备仿真模式的特性，可以通过在切换器内不停模拟主机与键盘/鼠标的通讯，来确保主机与外围设备之间的连续通信。该仿真功能使设备无需经过枚举的过程，从而减少了切换过程中设备通讯失败的可能性。

SW USB

2 路和 4 路输入 USB 切换器

主要特性 输入：2 个 USB B 型插座连接器. 输入：4 个 USB B 型插座连接器. 主机和外围设备仿真模式使设备在输入切换时保持通讯. USB 2.0 兼容. 4 个端口输出集线器，为每路输出提供 5V、500mA 电源. 型号; 产品说明; 产品编号. SW2 USB; 2 路输入 USB; 60-952-01. SW4 USB; 4 路输入 USB; 60-953-01. SW4 USB Plus; 带仿真功能的 4 路输入 USB; 60-954-01. Extron SW2 USB、SW4 USB 和 SW4 USB Plus 是 2 路和 4 路输入的 USB 切换器，允许在多个主机设备与多个 USB 外围设备之间进行切换。它们为在当今的视音频环境中实现兼容 USB 2.0 的切换提供了易于集成的解决方案。所具备的几种特性可确保轻松集成，包括端口状态指示、RS-232 直通、前面板安全锁定和多点控制。SW4 USB Plus 提供了主机和外围设备仿真模式的特性，可以通过在切换器内不停模拟主机与键盘/鼠标的通讯，来确保主机与外围设备之间的连续通信。该仿真功能使设备无需经过枚举的过程，从而减少了切换过程中设备通讯失败的可能性。 USB. USB specification; USB 2.0 compatible. USB data rates; Low speed (1.5 Mbps), full speed (12 Mbps), high speed (480 Mbps). USB input. Connectors. SW2 USB; 2 female USB type B. SW4 USB, SW4 USB Plus; 4 female USB type B. USB output. Connectors; 4 female USB type A. Control/remote — switcher. Serial control port; (1) RS-232, 3.5 mm captive screw connector, 3 pole (1) RS-232 pass-through, 3.5 mm captive screw connector, 3 pole. Baud rate and protocol; 9600 baud, 8 data bits, 1 stop bit, no parity. Serial control pin configurations. RS-232; 1 = Tx, 2 = Rx, 3 = GND. RS-232 pass-through; 1 = Tx, 3 = GND. Contact closure. SW2 USB; (1) 3.5 mm captive screw connector, 3 pole. SW4 USB, SW4 USB Plus; (1) 3.5 mm captive screw connector, 5 pole. Contact closure pin configuration. SW2 USB; 1 = input 1, 2 = input 2, 3 = GND. SW4 USB, SW4 USB Plus; 1 = input 1, 2 = input 2, 3 = input 3, 4 = input 4, 5 = GND. Program control; Extron Simple Instruction Set (SIS?). General. External power supply; 100 VAC to 240 VAC, 50-60 Hz, external; to 12 VDC, 1 A, regulated. Power input requirements; 12 VDC, 0.70 A. Temperature/humidity; Storage: -40 to +158 °F (-40 to +70 °C) / 10% to 90%, noncondensing Operating: +32 to +122 °F (0 to +50 °C) / 10% to 90%, noncondensing. Cooling; Convection, no vents. Mounting. Rack mount; Yes, with optional rack shelf. Furniture mount; Yes, with optional under-desk mounting kit. Enclosure type; Metal. Enclosure dimensions; 1.6" H x 8.6" W x 3.0" D (1U high, half rack wide) (4.1 cm H x 21.8 cm W x 7.6 cm D) (Depth excludes connectors.). Product weight; 2.0 lbs (0.9 kg). Shipping weight; 3 lbs (2 kg). Vibration; ISTA 1A in carton (International Safe Transit Association). Regulatory compliance. Safety; CE, c-UL, UL. EMI/EMC; CE, C-tick, FCC Class A, ICES, VCCI. MTBF; 30,000 hours. Warranty; 3 years parts and labor.